MSDS Number: 02353ET - 2	.   V

•'•>...

24 Hour Emergency Assistance: CHEMTEL (877} 276-7223 General Assistance Number:  (877) 276-7285 •

MATERIAL rDENTTTY: TEXACO Antifreeze/Coolant

Section 1

••PRODUCT COEDES:  02353

COMPANY ADDRESS:  Equilon Enterprises LLC, P.O. Box 4453, Houston TX  77210-4453.

Section 2

INGREDIENTS    '"            '                       -.••.-;,         ..'-.y:/              CAS#'	..CONCENTRATION

Antifreeze/Coolant                                                                                    ••                        Mixture	;'•	100     %weight

Ethtlene Glycol	.                .         .                              .                             .     107-21-1''	90-96.99     %weight

DeionizedWater                       .  .       -                                                                          7732-18-5	'2-4.99     %weight

.Dipotassium phosphate                                                                                                  7758-11-4	"^   1-2.99     %weight

SECTION-3 ^^'—^^^^^^^?

EMERGENCY OVERVIEW	" '   -'-—"—-' -     -   - ;' '- - - = - -    - -    —--_.-=- = - _

Appearance &. Odor: Fluorescent green liquid

Health Hazards: May be harmful or fatal if swallowed. Do not induce vomiting. May cause aspiration pneumonias. May

canse CNS depression.	.'

NFPA Rating (Health, Fire, Reactivity): 2, t, 0	^

Rating:Least - 0     Slight - 1      Moderate - 2     High - 3  ^Extreme - 4 = ~ " '- ~ " ~ - ''- - ~

Inhalation:

In applications where vapors (caused by high temperature) or mists (caused by mixing or spraying) are created, brcathin may cause a mild burning sensation in the nose, throat and lungs.

Eye Irritation:

May cause slight irritation of the eyes.   If irritation occurs, a temporary burning sensation, minor redness, swelling, and/c bhzrrcd vision may result.

Skin Contact:

May cause slight irritation of the skin. If irritation occurs, a temporary burning- sensation and minor redness and/or swellin may result. Other adverse effects not expected from brief skin contact.

•\ V In^estion:

Tins material may be harmful or fatal if swallowed. Ir.gcstion may result in vomiting; aspiration (breathing) of vomitus int lungs' must be avoided as even small'., quantities may result in 'aspiration pncumonitis. Contains cthylcne glycol and/o dicthylcne glycol which arc toxic when swallowed. A lethal dose for an adult is I ml per kilogram or about 4 ounces (I/ cup). Severe kidney damage can occur as a result of ingestion. May cause Central Nervous System (CNS) depression.

This material and/or components may cause the following effects: Developmental Toxicity

.;

; Primary Target Organs:	- .       M

.The following organs and/or organ systems may be damaged by overexposurc to this material and/or its components.

TCDCD AncUct«M/CooUne	MJOSl   0 11 J ) CT

Signs and Symptoms:

Irritation as noted above.  Aspiration pneumonitus may be evidenced by coughing, labored breathing and cyanosis (skin); in severe cased death may occur.  Early to moderate CNS despression may be evidenced by giddiness or dizziness and nausea.  In extreme cases, unconsciousness and death may occur.  Kindey damage may be indications changes in urine output or appearance, pain, upon urination or in the ‘lower bock or general edema (swelling from retention).  Liver damage may be indicated by loss of appetite, jaundice (yellowish skin and eye color), fatigue sometimes pain and swelling in the upper abdomen.

Aggravated Medical Conditions:

Pre-existing eye, skin, respiratory, liver and kidney disorders and may be aggravated by exposure to this product.

For additional health information, refer to section 11.

SECTION 4   FIRST AID MEASURES

Inhalation:  Move victim to fresh air and provide oxygen if breathing is difficult.  Get medical attention.

Skin:  Flush exposed area with water and follow by washing with soap if available.  If redness, swelling, pain and/or blisters occur transport to the nearest medical facility for additional treatment.

Eye:  Flush with plenty of water while holding eyelids open.  Rest eyes for 30 minutes.  If redness, burning, blurred vision or swelling occur, transport to nearest medical facility for additional treatment.

Ingestion:  DO NOT take internally.  DO NOT induce vomiting.  If vomiting occurs spontaneously, keep head below hips to prevent aspiration of liquid into lungs.  Get medical attention.

Note to Physician:  Ethylene Glycol (EG) and Diethylene Glycol (DEG) intoxication may initially produce behavioral changes, drowsiness, vomiting, diarrhea, thirst, and convulsions.  ED and DEG are nephrotoxic.  End stages of poisoning may include renal damage or failure with acidosis.  Supportive measures, supplement with hemodialysis if indicated, may limit the progression and severity of toxic effects.  For ETHYLENE GLYCOL POISONING, intravenous ethanol is a recognized antidotal treatment; other antidotal treatments also exist for ethylene glycol poisoning.

SECTION 5   FIRE FIGHTING MEASURES

Flash Point ( Method):  260 degrees F/126.67 degrees C (Pensky-Martens Closed Cap)

Extinguishing Media:  Use water fog, alcohol foam, dry chemical or carbon dioxide (CO2) to extinguish flames.  Do not use a direct stream of water.

Fire Fighting Instructions:  Material will not burn unless preheated.  Clear fire area of all non-emergency personnel.  Only enter confined fire space with full gear, including a positive pressure, NIOSH approved, self-contained breathing apparatus.  Cool surrounding equipment, fire exposed containers and structures with water.  Container areas exposed to direct flame contact should be cooled with large quantities of water (500 gallons water per minute flame impingement exposure) to prevent weakening of container structure.

SECTION 6  ACCIDENTAL RELEASE MEASURES

Protective Measures:  Wear appropriate personal protective equipment when cleaning up spills.  Refer to Section 8.

Spill Management:  Flush area with water to remove trace residue; place flush solution in proper container for recycle or disposal.  Should source of leak if safe to do so.  Dike and contain spill.

FOR LARGE SPILLS:  Remove with vacuum truck or pump to storage/salvage vessels.

FOR SMALL SPILLS:  Soak up residue with an absorbent such as clay, sand or other suitable material.  Place in non-leak container and seal tightly for proper disposal.

Reporting:  U.S. regulations require-reporting releases of this material to the environment which exceed the reportable quantity to National Response Center at (800) 424-8802.

SECTION 7  HANDLING AND STORAGE

Precautionary Measures:  Avoid heat, open flames, including pilot lights, and strong oxidizing agents.  Use explosion-proof ventilation to prevent vapor accumulation.  Ground all handling equipment to prevent sparking.  Avoid contact with eyes, skin and clothing.  Wash thoroughly after handling.

Container Warnings:  Keep containers closed when not in use.  Containers, even those that have been emptied, can contain explosive vapors.  Do not cut, drill, grind, weld or perform similar operations on or near containers.

SECTION 8  EXPLOSURE CONTROL/PERSONAL PROTECTION

Chemical 	Limit	TWA	STEL	Ceiling	Notation

Dipotassium phosphate	ACGIH TLV 	1mg/m3

Dipotassium phosphate	OSHA PEL	1mg/m3	3mg/m3

1989 (revoked)

Ethylene Glycol	ACGIH TLV			100mg/m3

Ethylene Glycol	OSHA PEL 1989 (revoked)		50 ppm

Exposure Controls – Adequate ventilation to control airborne concentrations below the exposure guidelines/limits.  Eye washes and showers for emergency use.

Personal Protection – Personal Protection Equipment (PPE) selections vary based on potential exposure conditions such as handling practices, concentration and ventilation.  Information on the selection of eye, skin and respiratory protection for use with this material is provided below.

Eye Protection:  Chemical Goggles – if liquid is likely, or Safety glasses with side shields

Skin Protection:  Use protective clothing which is chemically resistant to this material.  Selection of protective clothing depends on potential exposure conditions and may include gloves, boots, suits and other items.  The selection (s) should take into account such factors as job task, type of exposure and durability requirements.  Published literature, test data and/or glove and clothing manufacturers indicate the best protection is provided by Neoprene, or Nitrile Rubber.

Repiratory Protection:  If engineering controls do not maintain airborne concentrations to a level which is adequate to protect workers, approved respiratory must be worn.  Respirator selection, use and maintenance should be in accordance with the requirement of the OSHA Respiratory Protection Standard, 29 CFR 1910.134.  Types of respirator(s) to be considered in the selection process include:  for Mist:  Air Purifying, R or P style NIOSH approved respirator.  For Vapors:  Air Purifying, R or P style prefilter & organic cartridges, NIOSH approved respirator.  Self-contained breathing apparatus.

SECTION 9    PHYSICAL AND CHEMICAL PROPERTIES

Appearance & Odor:  Fluorescent green liquid, substance chemical family:  Ethylene Glycols

Boiling Point	226 degrees F		Flash Point	260 degrees F (Pensky-Martens Closed Cup)

Freezing Point	-32 degrees F		Odor	Mild odor

pH	10.2-11		Specific Gravity	1.12-1.14

NOTE:  The freezing and boiling values reflect a 50% solution in water at atmosphere pressure.

SEXTION 10    REACTIVITY AND STABILITY
Hazardous Decomposition Products:  Thermal decomposition products are highly dependent on combustion conditions.  A complex mixture of airborne solids, liquids and gases will evolve when this material undergoes pyrolysis or combustion.  Acids, Aldehydes, Carbon Monoxide, Carbon Dioxide, Kentones and other unidentified organic compounds may be formed upon combustion.

SECTION 11:  TOXICOLOGICAL INFORMATION

Acute Toxicity

Test	Result	OSHA Classification	Material Tested

Dermal LD50	.2 g/kg(Rabbit)	Non-Toxic		Based on components(s)

Carcinogenicity Classification

Chemical Name	NTP	LARC	ACHIH	OSHA

Antifreeze/Coolant	No	Not Reviewed	No	No

Development Toxicity:  Liver – prolonged and repeated ingestion of ethylene glycol has produced liver damage in rats.

SECTION 12  ECOLOGICAL INFORMATION

Environmental Impact Summary:  There is no ecological data available for this product.

SECTION 13    DISPOSAL CONSIDERTIONS

RCRA Information:  Under RCRA, it is the responsibility of the user of the material to determine, at the time of the disposal, whether the material meets RCRA criteria for hazardous waste.  This is because material uses, transformations, mixtures, processes, etc. material affect the classification.  Refer to the latest EPA, state, and local regulations regarding proper disposal.

SECTION 14  TRANSPORT INFORMATION

The Department of Transportation Classification:  This material is not subject to DOT regulations under 49 CFR Parts 171-180.  This material is not regulated under 49 CFR if in a container of 119 gallon capacity or less.  If shipped in a container of over 119 gallon capacity then the DOT information must be accompanied with RQ notation, or, an otherwise ‘Not Regulated’ product will be classified at Environmentally Hazardous (solid/liquid) N.O.S. Class 9, Packing group III unless the product qualifies for the petroleum exemption (49 CFR 171.8)

Hazardous Substance/Material RQ:	Ethylene glycol/5272-7043 lbs

International Air Transport Association

Hazard Class/Division:	9 (Miscellaneous)

Identification Number:  UN3082

Packing Group:  III

Proper Shipping Name:  Environmentally Hazardous Substance, Liquid, N.O.S.

Technical Name(s):  Ethylene Glycol

International Maritime Organization Classification

Hazard Class/Division:  9 (Miscellaneous)

Identification Number:  UN3082

Packing Group:  III

Proper Shipping Name:  Environmentally Hazardous Substances, Liquid N.O.S.

Technical Name(s):  Ethylene Glycol

SECTION 15  REGULATORY INFORMATION

Federal Regulatory Status

OSHA Classification:  Product is not hazardous according to the OSHA Hazard Communication Standard, 29 CFR 1910.1200

Comprehensive Environmental Release, Compensation & Liability Act (CERCLA):

Ethylene Glycol		RQ 5000 lbs	Reportable Spill => 5272.704265 lbs or 632.45 gal

Potassium hydroxide	RQ 1000 lbs	Reportable Spill => 133333.3333 lbs or 15992.96 gal

Sodium hydroxide	RQ 1000 lbs	Reportable Spill => 416666.66667 lbs or 49978.01 gal

Ozone Depleting Substances (40 CFR 82 Clean Air Act):  This material does not contain nor was it directly manufactured with any Class I or Class II ozone depleting substances.

Superfund Amendment & Reauthorization Act (SARA) Title III:  There are no components in this product on the SARA 302 list.

SARA Toxic Release Inventory (TRI) (313):  Ethylene Glycol

Toxic Substances Control Act (TRCA) Status:  All component(s) of this material is (are) listed on the EPA/TSCA Inventory of Chemical Substances.

Other Chemical Inventories:  Component(s) of this material is (are) listed on the Australian AICS, Canadian DSI, Chinese Inventory, Europe EINECS, Korean Inventory, Philippines PICCS

Stage Regulation

The following chemicals are specifically listed by individual states; other product specific health and safety data in other sections of the MSDS may also be applicable for state requirements.  For details on your regulatory requirements you should contact the appropriate agency in your state.

New Jersey Right-To-Know Chemical List:  Ethylene Glycol (0878)	90-96.99% weight	Special Hazard

Pennsylvania Right-To-Know Chemical List:  1,2-Ethanediol (107-21-1)	90-96.99% weight	Environmental Hazard

SECTION 16	OTHER INFORMATION

HMIS Rating (Health, Fire, Reactivity): 2,1,0

Revision#:2   Revision Date 07/24/2001

Revisions since last change (discussion):  This Material Safety Data Sheet has changed because Equiva Services LLC has implemented new software to generate the sheet.  There will be slight differences in the hazard and precautionary language as we incorporate the guidance contained in the ANSI MSDS standard (ANSI Z400.1-1998).  There are no significant changes to the health, safety or precautionary messages.  We encourage you to take the opportunity to reread the sheet and review the information contained.

SECTION 17	LABEL INFORMATION

READ AND UNDERSTAND MATERIAL SAFETY DATA SHEET BEFORE HANDLING OR DISPOSING OF PRODUCT.  THIS LABEL COMPLIES WITH THE REQUIREMENTS OF THE OSHA HAZARD COMMUNCIATION STANDARD 929 CFR 1910.12000 FOR USE IN THE WORKPLACE.  THIS LABEL IS NOT INTENDED TO BE USED WITH PACKAGING INGENDED FOR SALE TO CONSUMERS.

Product Codes:  02353	Texaco Antifreeze/Coolant

WARNING;

MAYBE HARMFUL OR FATAL IF SWALLOWED.  ASPIRATION HAZARD IF SWALLOWED – CAN ENTER LUNGS AND CAUSE DAMAGE.  MAY CAUSE CENTRAL NERVOUS SYSTEM DEPRESSION.

MAY CAUSE DAMAGE:  Kidney, Liver

HEALTH EMERGENCY	CHEMTEL  (877) 276-7283

ADMINISTRATIVE INFORMATION

COMPANY ADDRESS:  	Equilon Enterprises LLC, P.O. Box 4453, Houston, TX  77210-4453

Company Product Stewardship & Regulatory Compliance Contact:  Timothy W. Childs

Phone Number:  (282) 874-7708

MSDS FAX BACK Phone Number:  (877) 276-7285

THE INFORMATION CONTAINED IN THIS DATA SHEET IS BASED ON THE DATA AVAILABLE TO US AT THE TIME AND IS BELIEVED TO BE ACCURATE BASED UPON THAT DATA.  IT IS PROVIDED INDEPENDENTLY OF ANY SALE OF THE PRODUCT.  FOR PURPOSE OF HAZARD COMMUNICATION IT IS NOT INTENDED TO CONSTITUTE PRODUCT PERFORMANCE INFORMATION AND NO EXPRESS OR IMPLIED WARRANTY OF ANY KIND IS MADE WITH RESPECT TO THE PRODUCT.  UNDERLYING DATA OR THE INFORMATION CONTAINED HEREIN.  YOU ARE URGED TO OBTAIN DATA SHEETS FOR ALL PRODUCTS OU BUY, PROCESS, USE OR DISTRIBUTE AND ARE ENCOURAGED TO ADVISE THOSE WHO MAY COME IN CONTACT WITH SUCH PRODUCTS OF THE INFORMATION CONTAINED HEREIN.

TO DETERMINE THE APPLICABILITY OR EFFECT OF ANY LAW OR REGULATIO WITH RESPECT TO THE PRODUCT, YOU SHOULD CONSULT WITH YOUR LEGAL ADVISOR OR THE APPROPRIATE GOVERNMENT AGENCY.  WE WILL NOT PROVIDE ADVICE ON SUCH MATTERS, OR BE RESPONSIBLE FOR ANY INJURIES FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.  THE UNDERLYING DATA, AND THE INFORMATION PROVIDED HEREIN AS A RESULT OF THAT DATA IS THE PROPERTY OF EQIVA SERVICES LLC AND IS NOT TO BE SUBECT OF SALE OR EXCHANGE WITHOUT THE EXPRESS WRITTEN CONSENT OF EQIVA SERVICES LLC.